	[image: image49.jpg]zine=h=




	Bedienungs- und Betriebsanleitung des Portalkrans Picco
	



	
	Operation and Service Instructions
	[image: image50.jpg]zine=h=







	Gantry Crane Picco


	Description
	Gantry Crane Picco

	Article Number
	8010197

	Machine/Serial Number
	

	Manufacturing Year
	

	Carrying Capacity
	500 kg

	Length of the Running Rail
	3.500 mm

	Height Max.
	3.000 mm

	Clear height max..
	2.800 mm

	Clear width
	1.600 mm

	Total weight
	80 kg

	Adjustable leg with crank (optional)


[image: image49.jpg][image: image50.jpg][image: image1.jpg]



Picture 1 Overall View Gantry Crane Picco
Table of Contents 
21
Contact


22
Conformity Declaration „CE“


23
Symbols and Warning Instructions


23.1
Warning Instructions on the Gantry Crane Picco


33.2
Symbols in the Operation Instructions


34
Regulations for the use of the Gantry Crane Picco


34.1
Field of Application of the Gantry Crane Picco


34.2
Inappropriate Application


34.3
Safety Regulations


44.4
Machine Identification Plate


45
Assembly


55.1
Removing the transportation packaging


55.2
Scope of Supply


55.3
Assembly and Mounting Instructions


96
Operation Instructions


96.1
Lifting of Monument Plates with the Gantry Crane Picco


106.2
Transporting and Lowering of Monument Plates


117
Maintenance and Inspection


117.1
Maintenance


117.2
Inspection


128
Technical Drawings and Parts Listing


128.1
Parts List Gantry Crane Picco




1 Contact
For any further questions and/or individual orders, please contact Weha-Ludwig Werwein GmbH or the manufacturer directly:

	In Germany
	
	In Austria
	
	

	WEHA-Ludwig Werwein GmbH
	
	WEHA Steinbearbeitungsmaschi-nenvertriebsgesellschaft m. b. H.
	
	Visit us at our

	Post Box  13 55
	
	
	
	Internet Website:

	86331 Königsbrunn / Augsburg
	
	Lange Gasse 17
	
	

	Wikingerstr. 15
	
	A-1080 Wien
	
	http://www.weha.com

	86343 Königsbrunn / Augsburg
	
	
	
	

	Telephone: +49 (0) 8231 / 60 07-0
	
	Telephone: +49 (0) 1 / 40 39 340
	
	

	Telefax:      +49 (0) 8231 / 60 07-148
	
	Telefax:      +49 (0) 1 / 40 88 762
	
	E-Mail: info@weha.com

	E-Mail: info@weha.com
	
	E-Mail: office.weha@aon.at
	
	

	
	
	
	
	


2 Conformity Declaration „CE“
	EC-Conformity Declaration
98/37/EG



	Manufacturer:
	WEHA Sp. z o. o.

Ul.Armii Krajowej 31

Pl-58150 Strzegom

Telefon 0048 / 76 / 87 03 - 700

Telefax 0048 / 76 / 87 03 - 046


	Machine Type:
Gantry Crane Picco

Application:
For lifting plates of stone
Machine Number:



	Signature:
	[image: image2.jpg]"METAL-JAV/OR"Sp. z 0.0,
59-400 Jawor, ul. Wiej
1l. 076/370-37-00, f3x 076/
NIP 695-13-46-385, Id.





Herr Rajewski

	We hereby certify that the above mentioned machine corresponds to the requirements in accordance with the above guidelines.




3 Symbols and Warning Instructions
The following explanations of the symbols appearing on the machine or in the operating instructions should help you to understand them.
3.1 Warning Instructions on the Gantry Crane Picco
The following warning symbols can appear on the machine respectively unit:

	
[image: image3.wmf] 

 


	
	This symbol points out the availability of the related operating instructions and warns the operator to read these instructions prior to the installation or the commissioning of the machine. 

	[image: image4.png]



	
	This warning symbol appearing on the machine points out to the operator the existence of electrical current, which is strong enough to cause an electric shock to the operator upon contact with the danger point.  In such a location, health or  loss of life are at risk.

	[image: image5.png]



	
	This warning symbol denotes the need of the operator to wear ear-protection and eye goggles.

	[image: image6.jpg]=




	
	Warning as to suspended load!  This warning symbol appearing on the machine warns the operator that he is forbidden to walk or stand under suspended loads.

	[image: image7.jpg]



	
	Danger of bodily harm!  This warning symbol appearing on the machine warns the operator that during use of the machine, utmost attention must be used otherwise negligence could cause an accident

	[image: image8.jpg]>




	
	This warning symbol located on the unit or machine instructs the operator that the suspended load must be equally distributed.  Slanted and unevenly distributed loads may not be raised or transported!

	[image: image9.jpg]



	
	Warning of contact with high tension lines.  This warning symbol on the unit or machine warns the operator that danger may be caused by the operator in the event of negligent contact with high tension lines.

	[image: image10.jpg]000




	
	This mandatory sign obligates the operator of the machine to use and wear safety shoes, protective gloves and protection helmet.


3.2 Symbols in the Operation Instructions
The following symbols can appear in the operation instructions and are defined accordingly:

	[image: image11.png]



	
	This exclamation mark, framed in an equilateral triangle, draws the operator’s attention to the fact that very important directions are available and the operator should read and follow the information with particular interest.

	[image: image12.png]



	
	Obligation to recycle waste materials into the production process and to safeguard the natural resources and protect the environment.


4 Regulations for the use of the Gantry Crane Picco
Please pay particular attention to the information concerning the use of the machine.

	[image: image13.png]



	Attention!  Danger of Injury!!

Inappropriate use leads to personal injuries and damage to materials!!
	[image: image14.png]





This unit, respectively machine is especially conceived for one or several application fields.
The following chapter explains the possible applications and the possible dangers during the operation.

4.1 Field of Application of the Gantry Crane Picco
You have purchased a quality product of WEHA-Ludwig Werwein GmbH.
The Gantry Crane Picco is a helping device, to facilitate the easy transposing of objects, e.g. monument stones, and to assist you in your work.
Please note the following special characteristics:
· The Gantry Crane Picco is capable of being modified to become a transport vehicle.
· Total construction collapsible with a maximum length extension of 1.600 mm.
· Aluminum light-weight construction.
· Erection on uneven foundation by means of leveling adjustment.
· The Gantry Crane Picco can be varied in height and width.
4.2 Inappropriate Application
The Portal Crane Picco is to be exclusively for lifting and transporting of stone plates with a total maximum weight of 500 kg.!

Utilize the Gantry Crane Picco exclusively for the intended purpose and avoid materials, which could damage the individual construction components or the metal!
· The Gantry Crane Picco is complete and specifically designed for the proposed application.

· The Gantry Crane Picco may not be erected in a slanted position.

· The permissible carrying capacity of 500 kg may not be exceeded.

· The Gantry Crane Picco may not be erected on a soft foundation.

· The Gantry Crane Picco may not be erected on an incline, slope..

· The Gantry Crane Picco may not be pulled on a slant.

· The Gantry Crane Picco may not be used with a prevailing wind speed of 10 m/s.
· Never stand or walk under a suspended load!!
4.3 Safety Regulations
· Please read completely  these operating instructions before using the Gantry Crane Picco!

· Keep the operating instructions carefully in the proximity of the Gantry Crane Picco, for future reference or when in doubt!

· Unconventional changes or installing of additional devices will lead to the cancellation of any liability by the dealer or manufacturer!

· Remove all articles from the work area, which can be a possible source of danger!

· Only authorized persons may use the Gantry Crane Picco.  Each user must have read and understood this documentation.

· Before each use, check the basic parts and welding seams for damages!  In case of defects do not use the Crane.  The use by an unauthorized third party must be prevented by suitable measures!

· Make certain that the location where the Crane is to be assembled offers suitable foundation carrying capacity! 

· The supporting foundation must provide a carrying capacity of from 6.000 kg/m in the event no support plates are laid beneath the legs of the crane!!

· The maximum permissible carrying capacity 500 kg.

· Standing under a suspended load is absolutely  forbidden!!

· Carrying capacities may not be pulled on a  slant .

· The bolt for the height adjustment must be inserted at the peak of the side piece (A-Form)!  It is FORBIDDEN to permit the bolt for the height adjustment on the cross bridge of the side piece (A-Form) to be used  for supporting purposes!

· The Gantry Crane Picco may not be used with a prevailing wind speed of 10 m/s.

· All bolts must be safely secured to prevent unintentional loosening.

· Maintain a clean working area and provide for adequate lighting!

· Protective work clothing in accordance with prevailing instructions are  to be used when operating the Crane!

· Among other items belong the following:

· Protective Work Gloves in accordance with current safety measures to prevent accidents!

· Protective Work Safety Shoes in accordance with the current safety measures to prevent accidents!

· Use only suitable work pieces exclusively suitable for this Crane. Do not lift slabs liable to crack or break during work!

(“sound stone plates”).

· Check to ensure during use that no one is in the proximity of the Crane, in order to avoid danger to a third party!!

· Check and make certain that all parts are correctly installed!

· In case damages or malfunctions of the machine arise, please contact the dealer or manufacturer immediately!

· Protect the machine against environmental factors, e.g. rain, sunrays, humidity, etc.!

· In case of irreparable damage or other reasons which prevent the further use of the Crane for a long term, Stop operation by making the machine unusable thus making certain there is no danger to other persons!

· [image: image51.png]


If disposing of the machine or parts thereof; dismantle and separate the different materials which can then 

be disposed of in accordance with recycling regulations!

4.4 Machine Identification Plate
	The identification plate indicates the origin, identity as well as year of manufacture and, if necessary the maximum carrying capacity

Upon receipt of the goods, please compare the identification plate with the data on the first page of the operation instructions.  If the data differs from each other, please contact immediately the dealer or the manufacture
	[image: image15.png]Lieferer:

Bezeichnung:

Typ:
Fabrik-Nr.
Baujahr:
max. Tragfk.

PP D-86343 Kénigsbrunn

o>







5 Assembly
Please read carefully the information in the chapter „Assembly“ and follow it precisely.  The manufacturer or the Company WEHA-Ludwig Werwein GmbH will not be held liable for any damage due to incorrect assembly.

On receipt of the goods, check the supplied commodity for any damages incurred during transportation and packing!  Advise the Manufacturer/Agent respectively the Transport Company immediately of any damages!

	[image: image16.png]



	Any defects, caused by incorrect transport,  incorrect  assembly and operation are not covered under the Warranty by the dealer respectively the manufacturer!!
	[image: image17.png]





Pay attention that the protective covering of the metal parts will not be damaged by sharp edge articles!

Absolute attention must be devoted to the Safety Regulations in Chapter 3.3!

5.1 Removing the transportation packaging
The Gantry Crane is usually delivered in cardboard packaging. 

Prior to unpacking, please compare the attached delivery note with the supplied articles!

Remove the transportation packing:

Release the adhesive strips and/or strapping bands and unpack the contents very carefully!

Be careful when unpacking the Gantry Crane in order to prevent damages to the contents!

Strong blows to the building parts of the Gantry Crane on sharp edges can damage the surface and other sections!

Dispose of the packing waste for recycling in accordance with the local guidelines! 

5.2 Scope of Supply
Prior to beginning the assembly, please check precisely to see whether all necessary parts are available.  If the scope of supply is not complete, contact the dealer or the manufacturer immediately.

Scope of Supply consists of:
	1 x Operation and Service Instructions
1 x Gantry Crane Picco consisting of:
· 2 x Side pieces



                             [A1]

· 4 x legs for the side pieces


             [B1]

· 2 x supports for the Alu-Rail

                             [C1]

· 1 x collapsible Alu-track 


                             [D1]

· 1 beam trolley



                             [E1]

· 4 bolts for the leg adjustment 160 mm long
                             [G1]

· 2 x bolts for the height adjustment 170 mm long
             [H1] 

· 4 x bolts for the support brace 170 mm long
                             [H1]

· 2 x Grips for the height adjustment w/crosshead pin 155 mm long [I1]

· 1 x pulling carriage complete with steering rod
· 1 x chained geared pulley block (optional)

[image: image52.png]


[image: image53.png]




[image: image18.jpg]





Picture 4.2 – overall view hardware

	[image: image54.jpg]


[image: image19.jpg]



Picture 4.1 – Overall view Gantry Crane Picco
[image: image20.jpg]



Picture 4.1b – adjustable leg with crank (optional))


5.3 Assembly and Mounting Instructions
The Gantry Crane Picco is delivered in an unassembled condition.
The assembly can be accomplished by one individual.

· The floor space need not be horizontal (on the level) and not flat.
· As a matter of principle, the Gantry Crane Picco is delivered as individual components.
[image: image21.jpg]



Picture 4.3 Pulling carriage for the components Gantry Crane Picco
	· Shove the tracks (A4) of the transport vehicle axle to the left direction.
· Now lay the side pieces with the groove (B4) of the supporting legs on to the supporting surface of the transport vehicle in such a manner, that the axles can be laid down into the two grooves.

	[image: image22.jpg]



Picture 4.4 Gantry Crane Picco as transport vehicle

	· Now lay the second side part (A5) on top of the first side part.
· Next Step is to lay the collapsed tracks (B5) on very top.

· Now wrap the entire building component with the lashing strap.

· Now the Gantry Crane Picco can be driven to the desired building site.
	[image: image23.jpg]



Picture 4.5 Gantry Crane Picco as transport vehicle


	· Firstly open the lashing strap whereby the locking clamp (A6) is to be released .

· The tension lever (B6) is to be completely swung out so far as to permit the lashing strap to be pulled off.

· Lay the lashing strap to a secure location.
	[image: image24.jpg]



Picture 4.6 Release lashing strap


	· The tracks (A7) can be lifted by the grip (B7) and placed to the side.
	[image: image25.jpg]



Picture 4.7 Place the tracks to the side


	· The tracks with the beam trolley (A8) are now to be positioned. edgewise so that the rollers of the trolley are found on the under side.

· Now the safety bolt (B8) is to be removed and the bolt (C8) is to be pulled out.
· In the next Step, the upper track half is to be spread open.
· Then the bolt (C8) along with the safety bolt is to be inserted into the beam trolley.
	[image: image26.jpg]



Picture 4.8 Tracks with the upper side in downwards


	· Now the spreaded open  tracks (A9) are to be turned 180 °, so that the rollers (B9) of the trolley are now found on the upper side.
· By lifting of the lower side piece (C9) one can now remove the transport vehicle.
	[image: image27.jpg]



Picture 4.9 The tracks spread open and vehicle removed



	· Now the two clamping type sockets (A10) are to be removed from the side piece.
· Hereto the bolt (B10) is to be removed and the clamping type sockets extracted accordingly.

	[image: image28.jpg]



Picture 4.10 Set up the first side part and remove bolt


	· Now take the grip on the upper side piece and lift up on the edge.

· Now remove the safety ring (A11) from the two socket pins and then the socket pins (B11) are to be extracted.
	[image: image29.jpg]



Picture 4.11 Set up first side part and remove bolt


	· In the next step the upper face support is to be placed upwards and the socket pins are to be inserted into face support and secured with the safety clip.
· Now the side piece is to be raised as well as the lower face support.
· Then insert the socket pin (C11) through the lower face support and secure with the safety clip.

· Pull in completely once again the support (D11).
	[image: image30.jpg]



Picture 4.11 Spread open the face support and insert bolt



	· The two clamping type sockets ((A12) are to be so assembled that the fitting nipple [B12] be inserted into the circular reception slot [C12) of the support.
· Hereby at the same time the clamping mechanism (D12) opens, in which the clamp lever points upwards as indicated in the picture.
	[image: image31.jpg]



Picture 4.12 Insert the clamp type socket


	· In the next Step, the face supports (A13) are loosened, by means of removing the grips for the height adjustment (B13).
· Now lift up the second side piece, spread open and insert  the socket pin in the same procedure as with the first side piece.
	[image: image32.jpg]



Picture 4.13 release the face supports


	· Now the track is to be laid in and check in advance, if the clamping mechanism (C14) is in opened position.
· Now the clamping mechanism (C14) is to be folded back again and the somewhat extended track (as illustrated in picture D14) should then be clamped firmly.
Attention! The track must over extend the clamping mechanism! (D14).

· Proceed exactly with the second side piece.
	[image: image33.jpg]



Picture 4.14 Put on the clamping shoe and put in the rail


	Following description to this Picture 4.3b – leg with crank in the side piece is only required when the subject leg with crank (optional, see parts list) has also been purchased!


· The Gantry Crane must be aligned upon completion of the assembly.  Hereto, the leg with the crank can be used also as an option.  Which would be  additional equipment you can buy for your Gantry Crane

       (See parts list).

· The leg with the crank for side piece (B15) would then as a substitute be inserted into the laying side piece (A15) until the bolt for the leg (C15, 160 mm long) can be placed into the Bore (D15).  Thereto, the bolt for the leg adjustment (C15) should be completely  inserted.  With the safety clip the bolt (D15) will be prevented from slipping out unintentionally.

· According to the number of legs with cranks have been bought, one can replace the legs on the end points accordingly.
	[image: image34.jpg]



Picture 4.15 Leg with crank in side piece

	· The Gantry Crane Picco can now be placed in upright position and moved to the desired work area and the (optional) chained pulley block  hung onto the trolley.

· The two grips for the height adjustment (A16, 155 mm long) are to be inserted completely into the two supports (B16).  Attention must be made to the direction of the cross key bolt, thereby the same is completely inserted into the profile and the bolt is consequently completely inserted.

Now the grips can be turned somewhat, thereby preventing an unintentional  slipping out.
· The cross beam can now be changed in the height, which should be accomplished WITHOUT being under load.

· The identical height must be present on both pieces.

· Adjustment of the adjusting legs of the Gantry Crane Picco can be accomplished in the height, thereby the tracks of the Gantry Crane are laying level.
	[image: image35.jpg]



Picture 4.16 Gantry Crane Picco in assembled condition


	[image: image36.png]



	Caution! Danger! Without use of supporting ground plates at the adjusting legs; the foundation must accommodate a carrying capacity of from 6.000 kg/m² with the 3 m Alu-rail design!
	[image: image37.png]





· WITHOUT using supporting ground plates at the adjusting legs the foundation must accommodate a carrying capacity of from 6.000 kg/m² with the 4 m Alu-rail design, when the permissible carrying load is hung on the Gantry Crane.

· When the ground foundation does not accommodate the required carrying capacity, then 4 strong plates must be placed under the four supporting legs.  The four plates must be adequately stabile and large enough so that the plates under load do not become deformed.

	[image: image38.png]



	Caution! Danger!!

The cross beam must be aligned horizontally, therefore „on the level“!
	[image: image39.png]





· The cross beam of the Gantry Crane must be horizontally aligned, therefore „on the level“, not only in the cross direction as well as the length direction.  The supporting legs are to be adjusted to the small differences in the floor surface.  When greater differences in the floor surface are present, then the Gantry Crane may not be placed into use!

· In the event supporting legs with crank have been purchased, then the simple up and down cranking of the legs with crank will level off the Gantry Crane.

· The un-assembly of the Gantry Crane is accomplished in the reverse procedure.

6 Operation Instructions
This Chapter should provide you with information to better understand the Gantry Crane.  Please read carefully the operating instructions to obtain optimal service!

Please pay attention that the protective cover of the metal parts will not be damaged by sharp edges!

	[image: image40.png]



	Any defects, caused by incorrect assembly and operation are not covered under the Warranty by the dealer respectively the manufacturer!!
	[image: image41.png]





Strict attention should be given to the safety regulations stipulated in Chapter 4.3!!

6.1 Lifting of Monument Plates with the Gantry Crane Picco
	· The beam trolley is to be moved above the centre of gravity of the stone plate.  
· Now the load chain located on the left side (B1) is to be pulled down.  The load hook is then lowered.  This is to be repeated so long as the required lift is achieved.

[image: image42.png]


 Attention! Caution!  Never exceed the capacity of 500 kgs for suspension

        and  lifting!      
                                                                                                                          [image: image43.png]



· Now the stone plate with the carrying lashing straps is to be hung on the load hook.  The use of the lashing straps is in accordance with related operating instructions.

· Now the load chain located on the right side (C1) is to be pulled down.  The stone plate is then raised.  This is to be repeated so long as the required lift is achieved.

If possible, the load should only be raised a short distance from the ground level.

[image: image44.png]


 Attention!  Caution! Never stand or walk under a suspended load!! [image: image45.png]



	
[image: image46.png]


Picture 5.1 – Hand chained pulley block with trolley


6.2 Transporting and Lowering of Monument Plates
· If the suspended load is to be moved with the Trolley, then the cross beam must be checked to establish that it is “on the level”.  Thereby the suspended load on the Trolley will not sway independently.  The lifted load should only be raised until it is free from the ground level.

· The Trolley with chain geared drive and Crane can then be moved to the required work place.  It is important that the Trolley should be moved cautiously.  Any unnecessary abrupt starting and stopping can cause the load to become out of control and the possible blows to the side parts of the Gantry Crane could thereby lead to the destruction of the Crane.

	[image: image47.png]



	Attention Danger!

The beam trolley under load should be moved with extreme caution!
	[image: image48.png]





Now with the pulling down of the load chain on the left side of the pulley block; the load hook and suspended load will then be lowered.  This procedure to be repeated on the load chain until the suspended load is completely lowered to the ground.

7 Maintenance and Inspection
7.1 Maintenance
The Gantry Crane needs no regular maintenance.  Nonetheless the Crane must be inspected at least once yearly.

7.2 Inspection



Picture 7.1 Control label
The Gantry Crane is a load suspension device.  The Employer, who uses this device is obligated to arrange to have the Gantry Crane inspected within one years period by an authorized, competent person in accordance with § 40,  section 1, VBG 9a prescriptions.

The next date fort he inspection of the Gantry Crane is indicated in the Yellow Label „next inspection date“.

For example April 2002.

Repairs and welding (poisoning danger) may only be carried out by authorized personnel who possess the

knowledge and abilities to perform the same.

· You can have your Gantry Crane inspected by Messrs. WEHA-Ludwig Werwein GmbH in Königsbrunn.  Please ask us!

· All nuts and bolts are to be checked for proper fitting and periodically tightened.
· All welding seams should be checked for corrosion and cracking.

· All profiles should be checked for deformation and or damages..

You should handle your Gantry Crane very carefully .  Excessive demands on your Crane could lead to a shorter work life span.

8 Technical Drawings and Parts Listing
In the following chapter the spare parts for the Gantry Crane are outlined

Please note when ordering your replacement parts.  Only a complete indication of the required spare parts can guarantee a smooth processing of your spare parts order.  Always indicate with your orders the required quantities, the description of the parts, article number and if available the standard indication.

8.1 Parts List Gantry Crane Picco
	Pos.
	Quant.
	Description
	Standard Indication
	Material

	1
	2
	Side pieces with folding beams
	
	Alu

	2
	4
	Legs for side pieces
	
	Alu

	3
	1-4
	Leg with crank for side piece (optional)
	Art. Nr. VD080221
	

	4
	2
	Supports for Alu-track
	
	Alu

	5
	1
	Alu-track with 3 m length
	
	Alu

	6
	1
	Beam trolley
	
	Alu

	7
	1
	Hand chained pulley block (optional)
	
	

	8
	4
	Bolts with safety pin for horizontal swing of the side piece face supports 
	
	

	9
	4
	Bolts with safety pin for leg adjustment
	
	

	10
	2
	Bolts with safety pin for height adjustment
	
	

	11
	2
	Grips for height adjustment
	
	

	Gantry Crane Picco 

Art. Nr.: 8010197



�





� EMBED Word.Picture.6  ���





H1





I1





G1





E1





D1





C1





B1





A1





A1





B4





A4





B5





A5





B6





A6





A7





B7





C8





B8





A8





D9





C9





B9





A9





A10





B10





B11





A11





D11





B11





C11





A11





D12





C12





B12





A12





B13





A13





�





D14





C14





B14





A14





�





C15





D15





B15





A15





B16





A16





C1





B1





A1








	Seite: 2 von  ASK  \* MERGEFORMAT 15
	File:


D:\\Eigene Dateien\Bedienungsanleitungen\Portalkran Picco.doc

Revision/ Erstellt am:0 / 13.12.2002


	File: N:\Bedienungsanleitungen aktuell\WG800\Kräne\Operating Instr. - Gantry Crane Picco.doc
Version / Erstellt am: 01 / 10.05.2005/Operation Instructions Gantry Crane Picco
	Page 1 of 12



_1101377699.bin

_1175933742.doc


�












_1037025998.bin

_1046009088.unknown

_1006693911.doc
�



�
















