	
	Instruction manual of the Trolley TR 500 and WTR 750
	[image: image20.jpg]





	
	Instruction manual of the Trolley TR 500 and WTR 750
	[image: image21.jpg]






	Trolley TR 500 and WTR 750


[image: image1.jpg]


 [image: image2.jpg]



Picture 1 Overall view 

              Trolley TR 500                   Trolley WTR 750

	Description
	Trolley

	Type
	 FORMCHECKBOX 
 TR 500
	 FORMCHECKBOX 
 WTR 750

	Machine/ Serial no.
	

	Manufacturing year
	

	Trolley
	TR 500
	WTR 750

	Load capacity
	500 kg
	750 kg

	Net weight 
	21 kg
	32 kg

	Overall height
	1.210 mm
	1.260 mm

	Overall depth
	460 mm
	ca. 450 mm

	Overall width 
	630 mm
	650 mm


[image: image3.png]



Contents

21
Contact


22
Symbols and warning instructions


22.1
Warning instructions on the trolley


22.2
Symbols in the instruction manual


33
Regulations for the use of the trolley


33.1
Field of application of the trolley


33.2
Inappropriate use


33.3
Safety regulations


43.4
Machine identification plate


43.5
Technical and structural data


43.5.1
Trolley TR 500


43.5.2
Trolley WTR 750


54
Scope of supply and assembly


54.1
Scope of supply


54.2
Remove of the transportation package


54.3
Instructions for the assembly


55
Instruction manual


56
Maintenance and inspection


56.1
Weekly maintenance


66.2
Yearly inspection


67
Part list




1 Contact
For any questions and orders please contact the WEHA-Ludwig Werwein GmbH or directly the producer:

	In Germany
	
	In Austria
	
	

	WEHA-Ludwig Werwein GmbH
	
	WEHA Steinbearbeitungsmaschi-nenvertriebsgesellschaft m. b. H.
	
	

	PO Box 13 55
	
	
	
	

	86331 Königsbrunn/ Augsburg
	
	Lange Gasse 17
	
	

	Wikingerstr. 15
	
	A-1080 Wien
	
	http://www.weha.com

	86343 Königsbrunn/ Augsburg
	
	
	
	

	Phone: +49 (0) 8231 / 60 07-0
	
	Phone: +49 (0) 1 / 40 39 340
	
	

	Fax: +49 (0) 8231 / 60 07-148
	
	Fax: +49 (0) 1 / 40 88 762
	
	E-Mail: info@weha.com

	E-Mail: info@weha.com
	
	E-Mail: office.weha@aon.at
	
	

	
	
	
	
	


2 Symbols and warning instructions
The following warning symbols can occur on the machine.

2.1 Warning instructions on the trolley 

	


	
	This symbol points out the availability of the operating instructions and warns the operator to read these instructions before the installation or the commissioning of the machine. 

	[image: image4.jpg]



	
	This safety symbol obliges the operator to use and wear always feet, hand and head protection.


2.2 Symbols in the instruction manual
	[image: image5.png]



	
	This exclamation mark, framed in a equilateral triangle, draws attention to a very important information and recommends the operator to read and follow it with particular attention. The user has to pay particular attention to this note.

	[image: image6.png]



	
	Obligation to recycle waste materials into the production process to help safeguard the natural resources and save the environment.

	[image: image20.jpg]
	
	Marking of switches, articles, etc. in the pictures of this operating manual. The first number begins with the letter A, for the marking (e.g. article D) and the second number is the current numbering of the picture (e.g. picture 6.2) within a section e.g. 6.1 Inspection. The marking always refers to the accompanying or following description; otherwise the description refers to a marking in another picture.


3 Regulations for the use of the trolley
Please pay attention to the information about the use of the machine.

	[image: image7.png]



	Attention: danger of injury!

The inappropriate use leads to personal injuries!
	[image: image8.png]





The machine is especially conceived for one or several application fields. 

The following chapters explain the possible applications and the dangers during the use.

3.1 Field of application of the trolley
You’ve bought a quality product of WEHA-Ludwig Werwein GmbH.

The trolley is a tool enabling you to transport stone slabs and making work easier for you.
Please not the following characteristics:

· Stable steel construction and minimum net weight
· Easy changeable rubber coating (trolley TR 500)
· Easy changeable wood coating (trolley WTR 750)

· Safe and easy transport of stone slabs
· Lacquered surface
· Stable overall construction
3.2 Inappropriate use
The trolley may only be used to transport stone slabs up to a maximum weight of 500 kg (TR 500) or 750 kg (WTR 750).
· Only use the trolley for the scheduled tasks and do not use any materials which might damage the material or parts of the trolley.
· The trolley is especially and completely lined up for these fields of application

· Do not use the trolley as a work-table and do not process stone slabs on the trolley!
· Do not transport any work-pieces which might cause fire (e.g. heat)!
· Do not transport or store several stone slabs over each other.
· Do not transport too small, too big or too heavy work-pieces.
· The load of work-piece has to be equally distributed on the trolley.
· The trolley may not be used on an uneven surface.
· The trolley may not be used on inclined planes although it may be used on small ramps.
3.3 Safety regulations
· Read this instruction before using the trolley!

· Keep these instructions close to your trolley in case of any doubt!

· Unconventional changes or installing of additional devices will lead to the cancellation of any liability by the dealer or manufacturer!

· Remove all articles from the work area, which can be a possible source of danger!

· The trolley may only be used by authorized persons. Each user must have read and understood this documentation!

· Before each use check the basic parts and welding seams for damages! In case of defects don’t use the trolley; the use by unauthorized persons must be prevented by suitable measures!

· Take care that the area you are placing the trolley on has a suitable carrying capacity!

· The maximum loading capacity is 500 kg (TR 500) and 750 kg (WTR 750)!
· Only transport one work-piece at one time on the trolley!
· The load of work-piece has to be equally distributed on the trolley.

· The work-piece has to be fixed to the trolley during the transport (e.g. using a lashing strap)!
· The trolley may not be placed or moved on an uneven surface!
· The trolley may not be used on inclined ramps longer than 50 centimeters! The grade resistance corresponding to the loading has to be compensated by the user!
· Do not transport any too big or too small goods!

· Don’t stay under moving loads!

· Keep the working area clean and take care of suitable illumination!
· Always wear safety clothing while working with the trolley according to the rules for the prevention of accidents:
· Safety gloves and safety clothing according to the rules for the prevention of accidents!
· Shoes according to the rules for the prevention of accidents!
· Use work pieces exclusively appropriate to the machine like stone slabs without any cracks!

· Take care that no one is near the trolley while working with it, in order to avoid injuries!

· Check to see whether all parts are assembled correctly!

· If there is any damage to the trolley or any other malfunction contact immediately the manufacturer or the dealer!

· Protect the trolley against weather influences e.g. sun, humidity, rain or heat!

· In case of irreparable damage or other reasons, which prevent the use of the machine in the long-term, stop its operation, by making the machine unusable thus making sure that there’s no danger to other people!

· [image: image21.jpg][image: image22.wmf] 

 

 If disposing of the machine or parts thereof, dismantle and separate the different materials, which can then be
 disposed of to be recycled!

3.4 Machine identification plate
	The identification plate informs about the origin, identity as well as year of construction and, if necessary, about the maximum carrying capacity.
By the supply please compare the identification plate with the data on the first side of the operating instructions. If the data differ from each other, please contact immediately the dealer or the manufacturer.


	[image: image9.png]Lieferer: IELEI D-86343 Kbnigsbrunn

Bezeichnung:
Typ:
Fabrik-Nr.
Baujahr:

max. Tragfk.







3.5 Technical and structural data

Errors and technical changes expected without notice.
3.5.1 Trolley TR 500

	Overall height standing [mm]
	1.210 mm
	Maximum loading capacity [kg]
	500 kg

	Overall height lying  [mm]
	460 mm
	Size of the shovel [mm]
	450 mm x 200 mm

	Overall width [mm]
	630 mm
	Net weight [kg]
	21 kg

	Tires’ air pressure [bar]
	2,0 bar
	
	


3.5.2 Trolley WTR 750

	Overall height standing [mm]
	1.260 mm
	Maximum loading capacity [kg]
	500 kg

	Overall height lying  [mm]
	Ca. 450 mm
	Size of the shovel [mm]
	425 mm x 55 mm

	Overall width [mm]
	650 mm
	Net weight [kg]
	32 kg

	Tires’ air pressure [bar]
	3,8 bar -  4,0 bar
	
	


4 Scope of supply and assembly
Please read the information in this chapter very carefully and adhere most strictly to them. Damages caused by not taking care of these instructions are not to be compensated by the producer ort he WEHA-Ludwig Werwein GmbH.

4.1 Scope of supply
Check the delivered good for damages concerning packing. Damages caused by transport are to be reported immediately to the carrier and WEHA.

To the scope of supply belong:
· 1 x Instruction manual
· 1 x trolley complete
4.2 Remove of the transportation package
Normally the trolley is delivered in wood or cardboard packaging. 

Before unpacking please check the scope of supply. In case of missing parts, contact the trader or the producer immediately!

Remove of the transportation package: 
· Remove the tape or the strapping wire and unpack the contents carefully!
· Be carefully while unpacking the trolley not to damage it!

· Strong breaking of the construction units on sharp edges can damage the surface protection and other construction units!

· Dispose of the packing wastes according to the local guidelines and supply these to the material cycle! 

4.3 Instructions for the assembly
The trolley is send completely assembled. 
5 Instruction manual
To be able to work with your trolley in the best way, please pay attention to these instructions! Take care that the safety layer of the metal parts is not undamaged by sharp-edged devices!

	[image: image10.png]



	The dealer and/or manufacturer is not liable for any defects caused by wrong assembly!
	[image: image11.png]





Pay attention to the safety regulations in chapter 3.3!

	[image: image12.png]



	To move the trolley, the stone slab has to be lying onto the trolley safely or it has to be fixed on it using lashing straps!
	[image: image13.png]





· Take care that the tire’s air pressure is enough!

· Take care that the surface you are placing the trolley on provides enough carrying capacity!
· The maximum loading capacity of the TR 500 is 500 kg and the maximum loading capacity of the WTR 750 is 750 kg.
· Take care that the ground surface is horizontal and plane.
	[image: image14.png]



	Only move the trolley on a horizontal ground! Otherwise the user won’t be able to control the grade resistance!
	[image: image15.png]





· Move the trolley to the stone slab.
· Now lift the work-piece until it is completely over the trolley.
· Now tilt the trolley in order to raise the work-piece from the ground.
· Now move the work-piece to the required position
· Place the work-piece and lift it in order to pull the trolley off the work-piece.
6 Maintenance and inspection
	[image: image16.png]



	The maintenance of the trolley has to be executed only by a person having the necessary knowledge!

The producer is not responsible for damages caused by inobservance of these instructions!
	[image: image17.png]





6.1 Weekly maintenance
The trolley needs a regular maintenance.
· Pay attention to the safety regulations in chapter 3.3.!
· Check once a week whether to see the pressure of the tires is enough. The pressure of the tires of the trolley TR 500 it has to be 2.0 bar. Concerning the trolley WTR 750 it has to be between 3.8 and 4.0 bar.
6.2 Yearly inspection
The trolley needs an inspection once a year.
· Pay attention to the safety regulations in chapter 3.3.!

· Check your trolley once a year for rusty spots especially the welding seams!.

· Remove rust and protect this spot by putting a suitable anti-corrosive agent, e.g. spray of zinc.
· Check the rubber and wooden coatings for damages.

	[image: image18.png]



	Repairs and welding (danger of intoxication!) may only be done by persons having the appropriate knowledge and abilities!
	[image: image19.png]





7 Part list
In the following chapter there is a part list for the trolley.
Please be aware that when you want to order spare parts only a complete enumeration of the details concerning the spare part (see the spare parts list) can guarantee you a trouble-free processing of the order. Always indicate the quantity of spare parts, the name of the spare part, the article number and if necessary the abbreviation.

	Pos.
	Quantity
	Description
	Article/ Component number
	Tool

	1
	1
	Carrying element
	
	

	2
	2
	Rubber coating long for trolley TR 500
	Art. Nr. 8020476
	

	3
	1
	Rubber coating short for trolley TR 500
	Art. Nr. 8020475
	

	4
	2
	Wooden coating for trolley WTR 750
	
	

	5
	2
	Wheel 260 x 85, hub ø 25 x 75 mm for trolley TR 500 
	Art. Nr. 8020150
	

	6
	2
	Wheel 4.00-8, hub ø 25 x 75 mm for trolley WTR 750 
	Art. Nr. 8020290
	

	7
	
	
	
	

	Trolley
Trolley TR 500


Art. Nr. 8010090

Trolley WTR 750


Art. Nr. 8010103



D2





� EMBED Word.Picture.6  ���





� EMBED Word.Picture.6  ���








	Page: 6 of 6
	File: D:\\Bedienungsanleitungen\ Transportroller TR 500 WTR 750.doc

Version / Erstellt am: 1 / 19.09.2003


	File: D:\\Bedienungsanleitungen\Transportroller TR 500 WTR 750.doc

Version / Erstellt am: 1 / 19.09.2003
	Page: 1 of  ASK  \* MERGEFORMAT 6



_1030449439.doc
�



�
















_1153721789.doc
[image: image1.png]





�












_1030449437.doc
�



�
















